9:20 PM

4/26/2026

1001428105

Joel Pam Pam

Requirements Summary:

This project focuses on doing half precision floating point addition and subtraction based on the IEEE 754 standards. Inputs A and B are entered in as Hex values to represent the 16 bit binary numbers and the output is also displayed as a Hex value on the hex display.

Input Requirements:

- Input must be entered on the CSE 4 x 4 keyboard

- Input must be entered in binary IEEE format which can be done by entering in as a HEX value. For example, 3.75 in IEEE binary format is 0100001110000000 which should be entered in as 4380 on the keyboard

- Operand A should be entered by keying in the number and pressing key 2 on the Hex Board

- Operand B should be entered by keying in the number and pressing key 3 on the Hex Board

- Results R will be computed by pressing key 4 on the Hex Board

- Key 0 on the DE10 can be used to reset

Output Requirements:

- Operand A will be display on the Hex display in hex according to IEEE 754 half precision format

- Operand B will be displayed on the Hex display in hex according to IEEE 754 half precision format after pressing key 3

- Result R will be displayed on the Hex display in hex according to IEEE 754 half precision format after pressing key 4

- LEDR0 on the DE10 will showcase the Overflow Flag

- LEDR1 on the DE10 will showcase the Underflow Flag

Complete process to use the floating adder/subtractor on the DE10:

- Enter operand A in hex based on the IEEE 754 half precision format and press key 2. This will display A on the Hex display

- Enter operand B in hex based on the IEEE 754 half precision format and press key 3. This will display B on the Hex display

- Finally the Result will be displayed on the Hex display after pressing key 4 on the hex board.

Description of any added features:

No Additional Added features

-BONUS: Early Submission

-BONUS: Subtraction

-BONUS: Overflow and Underflow Flags

Organization Diagram:

Test Results:

- Summary:

| A | Hex value of A | B | Hex value of B | Result (R) | Hex value of R | Overflow | Underflow |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.75 | 0x4380 | 5.125 | 0x4520 | 8.875 | 0x4870 | 0 | 0 |
| 3.75 | 0x4380 | -5.125 | 0xC520 | -1.375 | 0xBD80 | 0 | 0 |
| -3.75 | 0xC380 | 5.125 | 0x4520 | 1.375 | 0x3D80 | 0 | 0 |
| 9.5 | 0x48C0 | 0.5625 | 0x3880 | 10.0625 | 0x4908 | 0 | 0 |
| 9.5 | 0x48C0 | -0.5625 | 0xB880 | 8.9375 | 0x4878 | 0 | 0 |
| -9.5 | 0xC8C0 | -0.5625 | 0xB880 | -10.0625 | 0xC908 | 0 | 0 |
| 125.125 | 0x57D2 | -12.0625 | 0xCA08 | 113.0625 | 0x5711 | 0 | 0 |
| 1000 | 0x63D0 | 1049 | 0x6419 | 2050 | 0x6801 | 0 | 0 |
| 65504 | 0x7BFF | 65504 | 0x7BFF | +∞ | 0x7C00 | 1 | 0 |
| -65504 | 0xFBFF | -65504 | 0xFBFF | -∞ | 0xFC00 | 1 | 0 |
| 0.0000610948 | 0x0401 | -0.00006103515625 | 0x8400 | +0 | 0x0000 | 0 | 1 |
| 0.000244140625 | 0x0C00 | -0.00024437904358 | 0x8C01 | -0 | 0x8000 | 0 | 1 |

- Demonstration

https://youtu.be/Am9OUKdBFNQ?si=iWIaEgJJ6qxy8XxN

SystemVerilog Code:

*Right Barrel Shifter,5bit Mux, 10bit Mux and Modular Exponent Subtractor are the same as the one’s used in HFPFAdder with the code presented above